import time

from board import SCL, SDA

import busio

from adafruit_servokit import ServoKit

import multiprocessing

import RPi.GPIO as GPIO

import os

import os

import sys

import time

import logging

import spidev as SPI

sys.path.append("..")

from lib import LCD_2inch

from PIL import Image,ImageDraw,ImageFont

from random import randint

touch_pin = 17

vibration_pin = 22

# Set up pins

GPIO.setmode(GPIO.BCM)

GPIO.setup(touch_pin, GPIO.IN)

GPIO.setup(vibration_pin, GPIO.IN)

# Raspberry Pi pin configuration for LCD:

RST = 27

DC = 25

BL = 18

bus = 0

device = 0

kit=ServoKit(channels=16)

servo=3

#Declare Servos

servoR = kit.servo[5]#Reference at 0

servoL = kit.servo[11]#Reference at 180

servoB = kit.servo[13]#Reference at 90

frame_count = {'blink':39, 'happy':60, 'sad':47,'dizzy':67,'excited':24,'neutral':61,'happy2':20,'angry':20,'happy3':26,'bootup3':124,'blink2':20}

emotion = ['angry','sad','excited']

normal = ['neutral','blink2']

q = multiprocessing.Queue()

event = multiprocessing.Event()

def check_sensor():

previous_state = 1

current_state = 0

while True:

if (GPIO.input(touch_pin) == GPIO.HIGH):

if previous_state != current_state:

if (q.qsize()==0):

event.set()

q.put('happy')

current_state = 1

else:

current_state = 0

if GPIO.input(vibration_pin) == 1:

print('vib')

if (q.qsize()==0):

event.set()

q.put(emotion[randint(0,2)])

time.sleep(0.05)

def servoMed():

servoR.angle = 90

servoL.angle = 90

servoB.angle = 90

def servoDown():

servoR.angle = 0

servoL.angle = 180

servoB.angle = 90

def baserotate(reference,change,timedelay):

for i in range(reference,reference+change,1):

servoB.angle = i

time.sleep(timedelay)

for j in range(reference+change, reference-change,-1):

servoB.angle = j

time.sleep(timedelay)

for k in range(reference-change, reference,1):

servoB.angle = k

time.sleep(timedelay)

def HandDownToUp(start,end,timedelay):

for i,j in zip(range(0+start,end,1),range((180-start),(180-end),-1)):

servoR.angle = i

servoL.angle = j

time.sleep(timedelay)

def HandUpToDown(start,end,timedelay):

for i,j in zip(range(0+start,end,-1),range((180-start),(180-end),1)):

servoR.angle = i

servoL.angle = j

time.sleep(timedelay)

def rotate(start,end,timedelay):

if start<end:

HandDownToUp(start,end,timedelay)

HandUpToDown(end,start,timedelay)

else:

HandUpToDown(end,start,timedelay)

HandDownToUp(start,end,timedelay)

def happy():

servoMed()

for n in range(5):

for i in range(0, 120):

if i <= 30:

servoR.angle = 90 + i #at 120

servoL.angle = 90 - i #at 60

servoB.angle = 90 - i

if (i > 30 and i <=90):

servoR.angle = 150 - i #at 60

servoL.angle = i + 30 #at 120

servoB.angle = i + 30

if i>90:

servoR.angle = i - 30 #at 90

servoL.angle = 210 - i #at 90

servoB.angle = 210 - i

time.sleep(0.004)

def angry():

for i in range(5):

baserotate(90,randint(0,30),0.01)

def angry2():

servoMed()

for i in range(90):

servoR.angle = 90-i

servoL.angle = i+90

servoB.angle = 90 - randint(-12,12)

time.sleep(0.02)

def sad():

servoDown()

for i in range(0,60):

if i<=15:

servoB.angle = 90 - i

if (i>15 and i<=45):

servoB.angle = 60+i

if(i>45):

servoB.angle = 150 - i

time.sleep(0.09)

def excited():

servoDown()

for i in range(0,120):

if i<=30:

servoB.angle = 90 - i #at 60

if (i>30 and i<=90):

servoB.angle = i + 30 #at 120

if(i>90):

servoB.angle = 210 - i

time.sleep(0.01)

def blink():

servoR.angle = 0

servoL.angle = 180

servoB.angle = 90

def bootup():

show('bootup3',1)

for i in range(1):

p2 = multiprocessing.Process(target=show,args=('blink2',3))

p3 = multiprocessing.Process(target=rotate,args=(0,150,0.005))

p4 = multiprocessing.Process(target=baserotate,args=(90,45,0.01))

p2.start()

p3.start()

p4.start()

p4.join()

p2.join()

p3.join()

def sound(emotion):

for i in range(1):

os.system("aplay /home/pi/Desktop/EmoBot/sound/"+emotion+".wav")

def show(emotion,count):

for i in range(count):

try:

disp = LCD_2inch.LCD_2inch()

disp.Init()

for i in range(frame_count[emotion]):

image = Image.open('/home/pi/Desktop/EmoBot/emotions/'+emotion+'/frame'+str(i)+'.png')

disp.ShowImage(image)

except IOError as e:

logging.info(e)

except KeyboardInterrupt:

disp.module_exit()

servoDown()

logging.info("quit:")

exit()

if __name__ == '__main__':

p1 = multiprocessing.Process(target=check_sensor, name='p1')

p1.start()

bootup()

while True:

if event.is_set():

p5.terminate()

event.clear()

emotion = q.get()

q.empty()

print(emotion)

p2 = multiprocessing.Process(target=show,args=(emotion,4))

p3 = multiprocessing.Process(target=sound,args=(emotion,))

if emotion == 'happy':

p4 = multiprocessing.Process(target=happy)

elif emotion == 'angry':

p4 = multiprocessing.Process(target=angry)

elif emotion == 'sad':

p4 = multiprocessing.Process(target=sad)

elif emotion == 'excited':

p4 = multiprocessing.Process(target=excited)

elif emotion == 'blink':

p4 = multiprocessing.Process(target=blink)

else:

continue

p2.start()

p3.start()

p4.start()

p2.join()

p3.join()

p4.join()

else:

p = multiprocessing.active_children()

for i in p:

if i.name not in ['p1','p5','p6']:

i.terminate()

neutral = normal[0]

p5 = multiprocessing.Process(target=show,args=(neutral,4),name='p5')

p6 = multiprocessing.Process(target=baserotate,args=(90,60,0.02),name='p6')

p5.start()

p6.start()

p6.join()

p5.join()